Adaptive Cruise Control for Vehicles

SEMESTER PROJECT

Student: Circiu Patrick-Sorin

Discipline: Knowledge-Based Systems

Contents

1.	Introduction	1

1.1.	General context	1

1.3.	Problem statement	2

2.	Dataset exploration and analysis	3

2.1.	Dataset overview	3

2.4.	Dataset Challenges	4

3.	Dataset pre-processing (1-5 pages)	5

3.1.	Dataset Organization and Pairing	5

3.2.	Resizing and Standaridzation	6

3.3.	Label Encoding	6

3.4.	Noise Handling and Data Variability	6

3.5.	Train-Validation-Test Splitting Strategy	6

3.6.	Summary of Dataset Pre-processing	7

4.	System modeling (1-10 pages)	8

5.	Conclusions (1-3 pages)	12

6.	Bibliography (1-2 pages)	14

# Introduction

## General context

Adaptive Cruise Control is a driver-assistance technology that helps a vehicle maintains a safe distance from the car ahead while keeping within a set speed. It automatically adjusts the vehicle’s speed, reducing the need for the driver to manually accelerate or brake.

Fully autonomous driving at SAE Level 4 or Level 5, targeted for model years 2021/2022, requires the integration of multiple sensor systems to ensure operational safety and reliability.

Current semi-autonomous driving systems utilize varying configurations and quantities of radar and camera sensors. However, high-resolution and cost-effective LiDAR (Light Detection and Ranging) systems with detection ranges of up to 300 meters remain in the pre-development phase. Most automotive manufacturers agree that achieving fully autonomous driving capability will require the combined use of radar, camera, and LiDAR technologies to provide system redundancy

With the growing emphasis on vehicle and passenger safety, the automotive industry is increasingly adopting active safety technologies such as Advanced Driver Assistance Systems (ADAS). These systems enhance driver awareness by detecting potential hazards and supporting collision prevention. Vision-based ADAS are widely implemented within automotive architectures. They typically consist of automotive camera modules, embedded vision platforms, and advanced image signal processing algorithms for object detection and scene interpretation.

While vision-based ADAS systems are effective at identifying objects within the camera’s field of view their performance is limited by visibility condition and optical constraints. They may experience reduced reliability in adverse environments such as heavy rain, fog, snow, mist, low-light conditions, dust, or nighttime driving.

In driving scenarios that require extended detection range, improved object recognition, and reliable operation in poor visibility conditions, radar-based ADAS systems provide a robust alternative. Radar technology enables object detection beyond the limitations of camera-based systems and maintains performance in challenging weather and lighting environments, making it critical component of advanced vehicle safety architectures.

  - Objectives

- Develop a vision-based object detection system

- Automatic speed adjustment

- Maintain safe following distance

- Collision avoidance

- Train an object detection model to identify relevant road objects, with emphasis on lead vehicle detection

- Evaluate the trained model using standard detection methods

## Problem statement

Road traffic accidents caused by inadequate following distance and delayed driver reaction remain a significant safety concern. Maintaining a safe gap between vehicles requires continuous attention, accurate distance judgment, and timely speed adjustments.

Adaptive Cruise Control (ACC) systems aim to mitigate this issue by automatically regulating vehicle speed based on the presence and behavior of a leading vehicle. However, implementing such a system requires reliable perception of the driving environment, accurate detection of vehicles in real-time, and stable estimation of relative distance and speed.

In this project, the challenge lies in developing a computer vision–based ACC prototype that uses sequential video frames to:

- Detect and identify the leading vehicle

- Track it across frames

- Estimate relative distance and motion

- Adjust the vehicle’s speed accordingly to maintain a safe following gap

# Dataset exploration and analysis

# Dataset overview

- The dataset used in this project consists of multiple driving video sequences, where each sequence is stored in a separate folder and decomposed into individual image frames.

- The dataset primarily contains road scenes captured from a forward-facing camera mounted on a vehicle. The key objects of interest for Adaptive Cruise Control (ACC) development are:

  - Vehicles

  - Trucks

  - Buses

  - Motorcycles

- Each image represents one time step in the driving sequence

- Each RGB image is paired with a corresponding semantic segmentation mask that provides pixel-level class annotations

  - Semantic classes

The dataset includes multiple semantic categories such as:

- Road surface

- Road line

- Vehicles

- Motorcycles

- Sidewalk

- Buildings

- Vegetation

- Sky (unlabeled)

- Pedestrians

- Traffic Sign

- Fence

- Wall

- Other

  - Spatial and Scene Characteristics

During dataset exploration, the following characteristics were observed:

- Urban driving environment

- Multiple vehicles per frame

- Perspective-based object scaling

- Clear lane markings

- Dynamic objects

  - Dataset Challenges

- Small objects (distant vehicles)

- Road and sky dominate pixel area

- Misclassification of vehicles

- Non-visible lane markings after training

  - Relevance to ACC System Development

The dataset supports the development of a vision-based ACC system through:

- Lead vehicle identification via vehicle-class segmentation

- Lane estimation using road and lane-marking segmentation

- Pixel-area-based distance approximation

- Scene understanding before control decisions

  - Summary

The dataset consists of RGB driving scenes paired with pixel-wise semantic segmentation masks. The segmentation labels provide detailed structural information about vehicles, lanes, and surrounding objects.

# Dataset pre-processing (1-5 pages)

In this project, pre-processing was designed to standardize the input format across all video-frame folders, ensure correct alignment between RGB frames and their semantic labels, and reduce noise and variability that can negatively affect segmentation training and downstream ACC decisions.

# Dataset Organization and Pairing

The dataset is organized into 2 folders, where each folder is organized into multiple subfolders. Each subfolder represents one driving sequence split into frames. Each RGB frame has a corresponding semantic segmentation label image with the same filename. During data loading, frames are paired with labels using realtive paths, ensuring that each input image is matched with its mask.

# Resizing and Standaridzation

To ensure consistent model input dimensions and stable training behavior, all images and labels are resized to a fixed resolution. In this project, the training resolution was set to 256x512 pixels. Standardizing resolution ensures:

- Reduced GPU/CPU memory usage

- Stable batch processing

# Label Encoding

Ground-truth segmentation labels are provided as color-coded RGB masks, where each semantic class corresponds to a specific RGB (Road, Car, Pedestrian etc.). Since the segmentation model requires integer class Ids for training, each RGB mask is converted into a 2D class-index mask

The conversion is performed by:

- Reading the label as an RGB image

- Packing RGB values into a single integer per pixel

- Mapping each packed value to a class index using a predefined color-to-index dictionary

# Noise Handling and Data Variability

Driving scenes inlude variability due to lightning, shadows, motion blur, and object scale. Instead of applying aggresive filtering that may remove small object (distant vehicles), the project relies primarily on:

- Input standardization (resize + normalization)

- Robust feature learning through the segmentation network

To reduce CPU overhead during training, converted label masks are cached in memory after first use. This improves training speed without changing the dataset content.

# Train-Validation-Test Splitting Strategy

Since the dataset consists of multiple folders, where each folder represents a full driving sequence, a sequence-level splitting strategy was adopted instead of a frame-level split. In video-based datasets, consecutive frames are highly correlated. If frames from the same video appear in both training and validation sets, the model may achieve artificially high validation accuracy due to temporal similarity rather than true generalizaiton. To avoid overfitting, the dataset was split at the folder level.

The data was split as:

- 70% training

- 15% validation

- 15% testing

# Summary of Dataset Pre-processing

The dataset pre-processing ensures standardized, reproducible, and model-compatible data preparation for semantic segmentation and ACC decision modelling

The following steps were implemented:

- Sequence-level dataset splitting to prevent data leakage

- Pairing of RGB frames with corresponding segmentation masks

- Resizing of all images and labels to a fixed resolution

- Conversion of RGB label masks into integer class-index tensors

- Normalization using ImageNet mean and standard deviation

# System modeling (1-10 pages)

The proposed Adaptive Cruise Control prototype is implemented as a two-stage system:

- Perception Module – Semantic Segmentation using a deep learning model

- Decision Module – Rule-based brake logic derived from segmentation output

  - System Architecture

The overall system consists of the following components:

- RGB Frame Input

- Pre-Processing (resize, normalization, label encoding)

- Semantic Segmentation Network (FCN-ResNet50)

- Semantic Mask Output

- Region-Of-Interest (ROI) Filtering

- Risk Assessment (vehicle, pedestrian, left-turn logic)

- Brake Decision Output

  - Perception Module: Semantic Segmentation

The perception module is implemented using:

- Fully Convolutional Network (FCN) with ResNet-50

- Pretrained weights from ImageNET

- Modified final classifier layer to output 13 semantic classes

The model performs multi-class pixel classification, where each pixel is assigned one of the predefined semantic labels (Road, Car, Pedestrian, etc.)

Output ∈ RCxHxW

  - Training Procedure

- Optimizer: AdamW

- Learning Rate: 1e-3

- Batch size: 4

- Epochs: 5

- Data resized to 256x512

Metrics used during training:

- Pixel Accuracy

- Mean Intersection over Union

  - Decision Module: ACC Brake Modeling

After segmentation, the predicted mask is analyzed to simulate ACC behavior. To restrict analysis to the relevant driving area, an ego-lane ROI is defined:

ROI=y1 :y2, x1 :x2				(4.1)

This region corresponds to:

- Lower central portion of the image

- Area directly in front of the vehicle

Instead of computing physical distance, a pixel-area ration is used as a proxy:

Carratio=Number of car pixels in ROITotal ROI pixels				(4.2)

If:

Carratio> ∋					(4.3)

Then the vehicle is considered too close.

  - Pedestrian Risk Detection

A stricter, smaller ROI in the close front-center region is defined. If pedestrian-class pixels are detected in the region:

Brake=1					(4.4)

  - Left-Turn and Oncoming Traffic Logic

The system includes:

- Road curvature detection via center-of-mass shift

- Oncoming traffic detection in a left-side ROI

If left-turning behavior AND left blinker is on AND oncoming vehicle detected:

Brake=1					(4.5)

  - Design Methodology

The design follows a modular structure:

- Separate perception from control logic

- Use semantic segmentation for detailed scene understanding

- Prioritize safety

The system was implemented incrementally:

- First validate segmentation performance

- Then integrate brake decision logic

- Finally test over full video sequences

  - Implementation and Deployment

- Programming language: Python

- Framework: PyTorch + Torchvision

- Visualization: OpenCV

- Operating system: Windows

- CPU-based training

  - Application usage

The system operates in two modes:

- Training mode

- Trains segmentation models

- Savel model weights

- Outputs predicted label images

- Simulation mode

    - Reads video folders

    - Processes segmentation masks

    - Displays:

      - Ego ROI

      - Pedestrian ROI

      - Oncoming ROI

      - “Brake” overlay

  - Discussion of Results

The ACC logic successfully:

- Detected close vehicles

- Identified approaching objects

- Triggered emergency breaking when pedestrians appeared

- Detected left-turn risk with oncoming traffic

Limitations include:

- Approximate distance estimation via pixel ratio

- Sensitivity to segmentation noise

- No physical speed control

  - Summary

The system integrates deep learning-based semantic segmentation with rule-based decision modeling to simulate an ACC prototype. The perception module provides pixel-level scene understanding, while the decision module interprets semantic region to trigger braking actions under risk conditions.

# Conclusions (1-3 pages)

  - Achieved Results

This project aimed to design and implement a prototype Adaptive Cruise Control (ACC) system using semantic segmentation and rule-based decision modeling. It was successfully trained on the provided dataset of driving sequences. The model achieved satisfactory performance in identifying dominant scene classes such as:

- Road

- Car

- Pedestrian

- Buildings

- Vegetation

The results showed that large and structurally consistent classes achieved higher IoU values, while smaller or less frequent classes showed lower IoU due to class imbalance and reduced pixel representation.

Using the predicted semantic masks, the decision module successfully simulated braking behavior based on:

- Lead vehicle proximity

- Relative approach estimation

- Pedestrian detection in a close-range region

- Left-turn detection with oncoming traffic verification

The system correctly triggered braking in scenarios involving:

- Close vehicles in the ego lane

- Rapidly approaching vehicles

- Pedestrians appearing in the driving path

- Left-turn scenarios with oncoming traffic risk

The use of Region-of-Interest(ROI) filtering significantly reduced false positives from vehicles in adjacent lanes or non-driving areas.

Compared to traditional ACC systems described in automotive literature, which typically rely on radar or LiDAR-based distance estimation, the proposed system uses purely vision-based semantic segmentation.

Advantages of the proposed approach:

- Pixel-level scene understanding

- Interpretable rule-based logic

- Low-cost vision-only design

- Modular structure

Limitations compared to industrial systems:

- Distance estimation is approximated using pixel ratios rather than real-world depth

- No continuous speed control

- Limited dataset size for deep learning training

  - Future Development Directions

- Continuous Speed Control

- PID-based speed control

- Smooth acceleration/deceleration profiles

- Adaptive time-gap control

- Real-World Deployment

  - GPU acceleration for real-time inference

  - Embedded implementation on edge devices

  - Integration into driving simulators

# Bibliography (1-2 pages)

[1] J. Long, E. Shelhamer, and T. Darrell, “Fully Convolutional Networks for Semantic Segmentation,” Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), pp. 3431–3440, 2015.

[2] “Volvo Pilot Assist with Adaptive Cruise Control,” AutoNation Volvo Cars San Jose. [Online]. Available: Volvo Pilot Assist with Adaptive Cruise Control | AutoNation Volvo Cars San Jose

[3] “What Is Adaptive Cruise Control?” [Online]. Available: (What Is Adaptive Cruise Control?).

[4] “Why Automotive Companies Should Adopt RADAR-based ADAS Systems.” [Online]. Available: (Why Automotive Companies Should Adopt RADAR-based ADAS Systems).

[5] “What Is Adaptive Cruise Control?” U.S. News & World Report. [Online]. Available: What Is Adaptive Cruise Control? | U.S. News

[6] “Three Sensor Types Drive Autonomous Vehicles,” Fierce Sensors. [Online]. Available: Three Sensor Types Drive Autonomous Vehicles | Fierce Sensors

[7] “What Is Adaptive Cruise Control, and How Does It Work?” ExtremeTech. [Online]. Available: What is adaptive cruise control, and how does it work? | Extremetech